# 802.11 - Wireless networks

Need for it:

Convenience
	mobility
	ease of setup
	scalability
	security
	cost

Wireless fundamentals

RF signals (radio frekvence)

EM wave propagation
		amplitude
		frequency
		wavelength
		phase

Reflection	()
	refraction	()
	scattering	()
	multipath	(viac cestne sirenie signalu)
	absorption	(absorbcia [prekazka kt signal pohlti] basically tlmi ten signal)

ak mame 2 transmitre 1 ma 1nW 2 ma 10nW
poviel je ako ho berieme ak T2 – T1 = 9mW ale T2/T1 = 10

Vediet tie hodnoty co tu chybaju!!!!!

Freee Path Loss:

Even without obstacles, a radio wave gets weaker as it moves away from the emitting source bcs the energy of the wave spreads

RSSI:

Bcs the RF wave might have been affected by obstacles in its path, it is 
important to determine how much signal is received by the other endpoint. The value that indicates the amount of power received is called received signal strength indicator (RSSI)

EIRP:

Effective isotropic radiated power
	actual power level radiated from the antenna
	regulated by governmental agencies

EIRP = Tx Power – Tx Cable + TX Antenna

Link budget:

Complete signal path from the transmitter to receiver with its gains and 
 		looses

Tx Power – Tx Cable + TX Antenna - Free space + rx antenna – rx calbe

Understanding power levels at the receiver:

EIRP ranges normally from 100mW (20dBm) to 1mW (0dBm)
		RSSI ranges usually fom 1mW (0dBm) don’t to 0mW (less than -100dBm)
		every receiver has a sensitivity lever or threshold that divides intelligible,  
 		 useful signals from unintelligible

SNR:

Signal to noise ration
		higher SNR is preferred

Regulatory bodies

ITU-R
	FCC
	ETSI
	IEEE
	Wi-Fi Aliance

NAJVIAC POUZIVANE KANALY SU 1,6,11  v teste !!!!
AKE FREKVENCIE POUZIVA WIFI (mHz alebo gHz)

Antenna characteristics

Radiation pattern ()
	gain
	beamwidth (ako sa posiela, directional, omnidirectional, isotropic)
	polarization (uhol v kt je proste ze antenky maju byt nastavene do hora)
	antenna types (omnidirectional, directional[patch, Yagi, dish],)

Wireless LAN topologies

BSS (basic service set) – group of wireless stations formed around the AP
	BSA – (basic service area) – wireless cell created around the AP
	BSSID – MAC address of AP wireless radio

VEDIET bssid!!!!!!!!!!

IBSS – independent basic service set – wireless network created by a stations 
 		without APs
	DS – distribution System – wired section of the network that can be reached through 
 		the AP
	ESS – Extended service set – links 2 or more Aps o one group

Wireless LAN topologies:

Repeater:

Extends the AP coverage
		dual radio can create dual half-duplex
		overlap of 50% required
		throughput impacted when single frequency used

Workgroup Bridge:

A WGB provides wireless connection from Ethernet port
		several devices can benefit if the WGB is connected to a switch

Outdoor wireless bridge:

Extend the LAN by linking LANs
		usually a few miles range
		point to point or hub and spoke

Mesh networks:

Provide wireless coverage over a large area
		devices are connected with redundant connection between nodes
		no single point of failure
		5GHz for backbone and 2,4GHz for users

RAP (root access point)
		MAP (mesh access point)

Wireless network standards features

802.11n

Channel aggregation (2 adjacent 20-MHz channels are combined)
		MAC aggregation packs smaller packets into a single unit
		improves throughput

802.11ac

5GHz only
		up to 8 spatial streams
		introduce up to 160MHz channel
		wave 2 introduce MU-MIMO

802.11ax

Introduce OFDMA – orthogonal frequency-division multiple access
		spatial frequency reuse (device coloring to diff own and neighboring devices)
		TWT – target wake time (signal trz nevisielaj [usetrenie baterie a umozni 
 			posielanie inych zariadeny bez rusenia])
		WPA3 (novsia forma zabezpecenia)