Implementing redundant supervisor engines in catalyst switches

## Recap:

Vrf, crf (virtualizacia)

Components HA:

- Redundancy

- Technology (hardware software features)

- People

- Processes

- Tools

Redundancy:

Vsetko dvojmo (napajanie klima, kable, ...)

Processes:

Should be correctly setted up, good documentation,

Optimal redundancy:

  - Alternative paths

  - Avoid too much redundancy

  - Avoid single point of failure

NSF with SSO

...

Routing protocols and NSF

...

No daisy chains

NEW i guess:

Stackwise access switches

- Supports recommended practice of using L3 connection between distribution switches witout having to use loopback cable or perform extra conf.

- Cisco catalyst 3750 switches in the access layer

- Much less complex than chains or stacks of other models

- Appears as one topology

## Implementing network monitoring

- Primarna motivacia je mat sistemy pod dohladom a vediet ci funguju dobre ak nefunguju tak o tom vediet skor ako zakaznik a spojazdnit to co najskor

Network management tools

- Syslog (system logging)

- SNMP (simple network management protocol)

- IP SLA (IP service level agreement) – to ze vieme nastavit neake tie vlastnosti siete

### Syslog

- Sys msg logging

- Enables device to report error and notification msg

- Uses UDP port 514

- These msges include severity level and facility

- Routers, switches, application sercers, firewalls, and other network appliances support syslog

Alternativne Sposoby su:

Su buffers, console (to ale je ten kabel co sa napoji v DC priamo tam moze byt neaky externi thou) tty (virtualny terminal) syslog server

Severity level su:

Lvl 0 – 7 (0 is worst possible severity)

System msg includes:

- Time stamp

- Facility: device that indicates the hardware device

- Severity: single-digit code

- Mnemonic: code that uniquely identifies the error msg

- Message-text: text string describing the condition

Basically vdaka syslog protokolu vieme posielat tieto veci na server

## SNMP

- It has 3 elements:

  - network management application (SNMP manager)

  - SNMP agents (running inside a managed device)

  - MIB database object that describes the information in predetermined format that the agent can use to populate the data.

- Je system s SNMP manager a na switchoch alebo co bezi snmp agent (sucast systemu) a komunikuje s manažerom a totok definuje ich komunikaciu

- SNMP defines how management info is exchanged between network management applications and management agents (to co je nad basically)

- SNMP version 1

- Zastarala, zmenena kôli bezpečnosti lebo vieš s tým aj manažovať

- Boly tam napr. Get request, set request, get response,...

- SNMP version 2

- Definuje neake nové správy ako get bulk request, inform request

- It deploys administrative framework defined in SNMPv1

- SNMP version 3

- It adds methods to ensure the secure transmission of critical data between managed devices

- It introduces 3 levels of security

In short SNMPv3 is recommended

Configuration of SNMP

- Step 1. configure SNMP access lists

- Step 2. conf SNMP community string

- Step 3. conf SNMP trap receiver

- Step 4. conf SNMPv3 user

So... narozdiel od syslog je tu objostranna komunikacia a neposielaju sa len logy na manažera.

## IP Service level agreement (IP SLA)

- Proaktivne získavanie informácii o neakých vlastnostiach sieti

- Veci do teraz sme zbierali len informacie alebo štatistiky, kým tu vieme vygenerovať testovaciu prevádzku.

- Je to akoby testovacia prevádzka pomocou kt viem zistiť info na vyše (či je apka online, jej ping, ci vsetko bezí ako má a vieme vytvoriť štatistiky...)

- Contract between service provider and customers

- Specifies connectivity and performance agreements

- Includes guaranteed level of network availability, network performance in terms of round-trip time, and network response in terms of latency, jitter, and packet loss

IP SLA opeation with responder

- Network manager configure IP SLA opearation by defining a target device, protocol, and port number on IP SLA source.

- Network manager can also configure reaction conditions. Opearation is scheduled to be run for a period of time to gather statistics.

IP SLA responder timestamps

- They are used in round-trip calculations

- When its send the packet includes T1 time after its received it adds TIME 2 and transmitted time T3

## netflow

Tok je teoreticky definovany s cieľom, číslom portu a zdrojom.

A na zistenie co v sieti v skutocnosti preteka sa pouziva toto.