HA (high availability)

Components:

- Redundanci (nadbytočnosť)

- Technology (hw and software features)
 	Failover – nech sa to automaticky prepne pri vade.

- People (technologicky vyuceny personal na mitigovanie human error)

- Processes (procesy ako change processes, snow, human processes)

- Tools

Redundancy:

- Dual wan

- Geographical location

- Secondary power

- Dual networking

- Dual processing servers

- Dual cooling

- Different ways roads where wew have the dc cables from 
(not all cables from one place)

Technology:

- Cisco nonstop forwarding (NSF)

- Stateful Switchover (SSO)

- Graceful restart

- Cisco IOS IP service level agreement (SLA)

- Object tracking

- Firewall stateful failover 
(vnima a sleduje aktualne a nove sessions, a zabezpecuje to ze ten node co caka ma sync session s tim prvim nodom a ked jeden spadne druhy iba prevezme existujuce sessions toho prveho)

People:

- PPDIOO: prepare, plan, design, implement, operate and optimize

- Work habits and attentio to detail important

- Skills are acquired via ongoing technical training

- Good communication and documentation critical

- Use lab testing and simulate

- Take time to design

- Identify responsibilities

Processes:

...

Tools:

- Network diagrams

- Documentation of network design evolution

- Key addresses, VLANs, and servers documented

- Documentation tying services to applications and physical servers

Network Level Resiliency:
-> ak nám spadne napr. nejaká sieť, tak musíme všetkým oznámiť (napr. routerom) že tá site je down
a aktualizujem im routovacie tabulky
Chceme aby prechodovy medzistav bol co najkratsi

Relies on monitoring with NTP, SNMP, syslog, and IP SLA

High availability and failover times

- Tuned routing protocols failover in less than 1s

- RSTP converges in about 1s

- EtherChanngel faiover approximately 1s

- HSRP 3s for hello and 10s for hold time

- Stateful service modules typically failover in 3-5s

- TCP/IP up to 9s tolerance

Optimal redundancy:

- Provide alternate paths

- Avoid too much redundancy

- Avoid single point of failure